Details

Bluetooth                                                                                                       Arduino

| EN | 3.3V |
| --- | --- |
| +5V | 5V |
| GND | GND |
| TX | 10 |
| RX | 11 |

AT command mode-

AT

AT+ADDR?     // Note the address of slave and master.

AT+ROLE?     // ROLE=0 i.e. it is slave; Role=1 i.e. it is master

AT+ROLE=n   // here n=0 means to change rile to slave, n=1 means to change role to master.

AT+UART?     // To check baud rate (should be same for master and slave)

(use same rate for transferring data ) prefer 38400

AT+UART=38400,0,0     // To change baud rate

AT+BIND=n    // n is address ex- 0021,13,0039FC (commas used in master AT command)

Ex- 0021:12:0039FC (colon used in slave command)

Now get exit from command mode.

| Button 04 and 02 -of Nano board | Button 05v, 02 of Arduino board |
| --- | --- |
| Vibrator (redwire)08 and (black wire) GND - Nano board | BUZZER - Vcc to 8 pins of Arduino and GND to GND of Arduino and GND of buzzer to I/O of buzzer. |